CONTROL TEAM

# Shared Kinematics Package
Usage Guide

UR5 Reference Robot • MATLAB / Robotics System Toolbox

Purpose

Explain how the Control team should use the shared robot configuration and MATLAB robot build to develop FK, IK, and Jacobian algorithms that can be independently validated by the Simulation team.

Core rule: Use the shared RobotConfig as the single source of robot parameters. Use the MATLAB robot build for visualization and debugging, but implement FK, IK, and Jacobian independently.

Version 0.1  |  Reference model: UR5  |  Status: Control handoff

# 1. What You Receive

The shared package is intentionally small. It gives Control the robot definition and a common visualization model without exposing Simulation’s independent validation implementation.

Shared_Kinematics_Package/
├── +config/
│   └── UR5.m
├── +robotmodel/
│   └── buildRobot.m
└── docs/
    ├── RobotModelUsageGuide.docx
    └── KinematicsInterface.md

| File | Purpose |
| --- | --- |
| `+config/UR5.m` | Canonical robot parameters and conventions. This is the source of truth. |
| `+robotmodel/buildRobot.m` | Builds the same robot as a MATLAB `rigidBodyTree` for visualization, frame inspection, and debugging. |
| `KinematicsInterface.md` | Defines the exact FK/IK/Jacobian interfaces, units, frame conventions, and deliverables. |

# 2. Start MATLAB in the Shared Package

Open MATLAB with the project/package root as the current folder, or add the package root to the MATLAB path. Do not add the `+config` or `+robotmodel` folders individually; MATLAB resolves namespace folders through their parent directory.

clear
clc
close all

% Load the shared robot definition
robot = config.UR5();

## Check that MATLAB can find both shared functions:

which config.UR5
which robotmodel.buildRobot

Expected result: Both commands should resolve to the files inside the shared package. If either returns “not found,” fix the project path before continuing.

# 3. Using the Robot Configuration

`config.UR5()` returns a MATLAB structure named `robot`. The Control team should read all robot-specific parameters from this structure instead of copying dimensions into its own functions.

robot = config.UR5();

robot.dof
robot.joints.names
robot.kinematics.a
robot.kinematics.d
robot.kinematics.alpha
robot.kinematics.thetaOffset
robot.limits.qMin
robot.limits.qMax
robot.tool.T_F_TCP

## 3.1 Joint-vector convention

All joint configurations are 6×1 column vectors in radians, ordered from Joint 1 to Joint 6.

q = [
    q1
    q2
    q3
    q4
    q5
    q6
];

% Mapping:
% J1 -> q(1)
% J2 -> q(2)
% J3 -> q(3)
% J4 -> q(4)
% J5 -> q(5)
% J6 -> q(6)

## 3.2 Reading DH parameters

a           = robot.kinematics.a;
d           = robot.kinematics.d;
alpha       = robot.kinematics.alpha;
thetaOffset = robot.kinematics.thetaOffset;

The shared convention is Standard Denavit-Hartenberg:Aᵢ = RotZ(θᵢ) · TransZ(dᵢ) · TransX(aᵢ) · RotX(αᵢ)

Do not hard-code: Avoid writing values such as `a2 = -0.425` inside Control algorithms. Read the parameters from `robot`. This lets the same algorithms later run on `config.WeldingRobot()` without rewriting robot-specific constants.

## 3.3 Joint limits and TCP

qMin = robot.limits.qMin;
qMax = robot.limits.qMax;

T_F_TCP = robot.tool.T_F_TCP;

For the current UR5 reference model, `T_F_TCP = eye(4)`, so the flange and TCP frames are coincident. The final welding robot will later contain a non-zero flange-to-TCP transform for the TIG torch.

# 4. Building and Visualizing the Shared Robot

Build the MATLAB reference geometry from the same RobotConfig:

robot = config.UR5();
robotModel = robotmodel.buildRobot(robot);

## Inspect the body/joint hierarchy:

showdetails(robotModel)

## Visualize the zero configuration:

q = zeros(6,1);

figure
show(robotModel, q, ...
    "Frames", "on", ...
    "Visuals", "off");

axis equal
grid on

## Visualize a general configuration:

q = deg2rad([
     30
    -45
     60
     20
    -30
     45
]);

figure
show(robotModel, q, ...
    "Frames", "on", ...
    "Visuals", "off");

axis equal
grid on

## Use this model to:

Check the robot topology and link sequence.

Verify the positive rotation direction of each joint.

Inspect Base, link, Flange, and TCP frames.

Debug whether a calculated FK/IK solution is physically reasonable.

Visualize candidate joint configurations during development.

# 5. What the Control Team Must Implement

The shared robot files define the robot. They do not replace the Control team’s kinematic algorithms. Control should implement three independent functions.

## 5.1 Forward Kinematics

function T_B_TCP = controlFK(robot, q)

    % Read shared robot parameters
    a     = robot.kinematics.a;
    d     = robot.kinematics.d;
    alpha = robot.kinematics.alpha;

    % Implement Control-team FK here

end

The output `T_B_TCP` must be a 4×4 homogeneous transform representing the TCP pose expressed in the Base frame.

## 5.2 Inverse Kinematics

function [qSolution, info] = controlIK(robot, T_B_TCP_target, qSeed)

    % Implement Control-team IK here

end

For the first version, one valid 6×1 joint solution is sufficient. The solution must satisfy the shared joint limits.

## 5.3 Jacobian

function J = controlJacobian(robot, q)

    % Implement Control-team geometric Jacobian here

end

The agreed project ordering is:[ v ; ω ] = J(q) q̇

Rows 1–3 are TCP linear velocity; rows 4–6 are TCP angular velocity. Both are expressed in the Base frame.

# 6. Critical Independence Rule

Important: The MATLAB `rigidBodyTree` is a shared visualization/debugging model. It must not become the submitted Control implementation of FK, IK, or Jacobian.

For example, the following is not acceptable as the Control FK implementation:

function T_B_TCP = controlFK(robot,q)
    robotModel = robotmodel.buildRobot(robot);
    T_B_TCP = getTransform(robotModel,q,"TCP");
end

Why? Simulation will use MATLAB’s reference kinematic functions independently to validate the Control algorithms. If Control uses the same functions, both sides are calculating the answer with the same engine and the comparison no longer provides independent verification.

## The intended separation is:

| Simulation | Control |
| --- | --- |
| MATLAB `rigidBodyTree` reference model | Independent FK / IK / Jacobian implementation |
| Reference FK / Jacobian / IK checks | Uses shared RobotConfig parameters |
| Validation and error metrics | May use shared robot model for visualization/debugging |

# 7. Recommended Development Workflow

| Step | Action |
| --- | --- |
| 1 | Load `robot = config.UR5()` and inspect the configuration. |
| 2 | Build `robotModel = robotmodel.buildRobot(robot)` and verify the frames/joint directions. |
| 3 | Implement `controlFK(robot,q)` using the shared parameters. |
| 4 | Test FK at zero, simple single-joint rotations, and several general configurations. |
| 5 | Implement `controlJacobian(robot,q)` using the same conventions. |
| 6 | Implement `controlIK(robot,T_B_TCP_target,qSeed)` and enforce joint limits. |
| 7 | Send the three functions to Simulation for independent validation. |

# 8. What to Send Back to Simulation

When the first kinematics implementation is ready, return:

`controlFK.m`

`controlIK.m`

`controlJacobian.m`

A short note describing the method used for FK, IK, and Jacobian.

## Suggested method note:

Forward Kinematics: Standard DH transformation multiplication
Inverse Kinematics: Analytical / numerical method
Jacobian: Geometric Jacobian derived from joint axes and positions

If you suspect a shared parameter is wrong: Do not quietly modify the RobotConfig in your own copy. Report the issue so Simulation and Control can review and update the shared configuration together.

# 9. What Simulation Will Validate

Simulation will use the same RobotConfig but an independent MATLAB reference model to check the Control implementation.

| Control output | Simulation validation |
| --- | --- |
| FK | Same joint vector `q` → compare TCP position and orientation. |
| IK | Take Control’s `qSolution` → run reference FK → compare achieved pose with requested target and check joint limits. |
| Jacobian | Same joint vector `q` → compare the agreed 6×6 geometric Jacobian convention. |

# 10. Transition to the Welding Robot

The current UR5 is a reference robot for verifying that the algorithms and interfaces work correctly. Simulation will separately determine the actual welding robot geometry using workspace, reachability, singularity, manipulability, and task requirements.

## The intended transition is:

% Reference stage
robot = config.UR5();
T = controlFK(robot,q);

% Later: actual project robot
robot = config.WeldingRobot();
T = controlFK(robot,q);

Design objective: Robot-specific parameters should change through `RobotConfig`; the Control kinematics interfaces should remain unchanged.

# Quick Reference

| Task | Command / Interface |
| --- | --- |
| Load robot | `robot = config.UR5();` |
| Build shared MATLAB model | `robotModel = robotmodel.buildRobot(robot);` |
| Inspect model | `showdetails(robotModel)` |
| Visualize | `show(robotModel,q,"Frames","on")` |
| Control FK | `T_B_TCP = controlFK(robot,q)` |
| Control IK | `[qSolution,info] = controlIK(robot,T_B_TCP_target,qSeed)` |
| Control Jacobian | `J = controlJacobian(robot,q)` |