Once per MATLAB session to add to path:

cd('D:\GitHub_Repos\Graduation_Project\Simulation-Control-GP\Simulation-ControlGP')

addpath(genpath(fullfile(pwd,'Final_Architecture')))

For Workspace Analysis:

reachable_workspace

collision_free_workspace

manipulability_workspace

usable_workspace

Improvements to Workspace Analysis

Random sampling can miss exact extremes

Sobol sampling spreads configurations well, but it may not sample the exact joint-limit boundaries or the exact pose producing maximum reach.

Later, we should add a boundary search that deliberately finds:

- Maximum and minimum X

- Maximum and minimum Y

- Maximum and minimum Z

- Maximum radial reach

- Minimum radial reach

This can use optimization in addition to sampling.

Manipulability Improvements

The current inverse-condition score is:

\[ \mu=\frac{\sigma_{\min}(J_v)}{\sigma_{\max}(J_v)} \]

This is a sensible dimensionless measure of linear TCP directional balance. However, it does not tell the entire story.

A configuration can theoretically have a reasonable ratio while all singular values are relatively small. Therefore, we may later retain:

- Inverse condition number: how balanced movement is.

- Minimum singular value: how weak the worst movement direction is.

- Manipulability ellipsoid: which Cartesian directions are easy or difficult.

The current map assigns each Cartesian voxel its best sampled manipulability. That answers:

Is there at least one good configuration for reaching this region?

That is useful but optimistic. Later, we could let you select:

- Best value

- Mean value

- Worst value

- 10th percentile

The lower percentile would show areas that are reliably good across multiple configurations.

Also, the current threshold of 0.10 is a preliminary engineering choice. It should eventually be justified by examining known good poses, near-singular poses, and welding trajectories.

Orientation And Dexterity

The current reachable workspace only answers:

Can the TCP position reach this location in at least one orientation?

For welding, orientation matters. A point may be reachable while the torch cannot approach it at the required angle.

A future dexterous-workspace analysis should test several orientations at every voxel and calculate:

\[ \text{orientation coverage} = \frac{\text{reachable tested orientations}} {\text{total tested orientations}} \]

This produces results such as:

- Reachable in one orientation

- Reachable in several orientations

- Fully dexterous for the tested orientation set

- Reachable with the required welding orientation

Task-specific torch orientation analysis should happen once the welding tasks and workpiece placement are better defined.

seful Visualizations

Future workspace outputs could include:

- XY, XZ, and YZ cross-sections

- Workspace slices at selected heights

- Outer workspace boundary surfaces

- Manipulability histograms

- Percentage of workspace above the threshold

- Workpiece positioned inside the workspace

- Side-by-side robot-design comparison

For design comparisons, useful summary values would be:

Maximum reach

Reachable volume

Usable volume

Usable/reachable volume percentage

Median manipulability

Low-manipulability percentage

Required-task coverage

So my recommended order from here is:

1. Keep the current workspace implementation.

2. Complete collision analysis.

3. Connect collision rejection to the workspace pipeline.

4. Add sample and voxel convergence testing.

5. Add optimized boundary searches.

6. Add orientation coverage when the welding task is defined.

7. Add cached results and robot-design comparison reports.